# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Medical Equipment

# Lot 1: Equipment for BLS and ILS

# Lot 2: Equipment for Hospital Patient Management

Publication reference: CN1-SO 1.2-SC002/4

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
|  | LOT 1. EQUIPMENT FOR BLS AND ILS |  |  |  |
| I.1 | Automatic External Defibrilator, Qty 10 Fully automatic public access defibrilator with pulsed biphasic waveform Disposable Li-MnO2 lithium cell 12 V, 2,8Ah Battery capacity: more than 180 shocks at maximum energy Dimensions (h x w x l): 70x230x220mm +_5% Weight: max. 1,6 kg (incl. battery) LCD screen with text and simbol display with minimal dimensions 100x37mm Energy levels for adults in minimal range of: 1 to150 J Information data store up on SD card: Recorded data: -ECG signal recording (minimum 2h) -Ambient noise recording (minimum 2h) -Event recording (minimum 500 events) Option for automatic data transmission : Intervention data and the device configuration to be sent to a server via an Ethernet network To be delivered with minimum one set of preconected disposable adult pads and carrying bag |  |  |  |
| I.2 | BLS training mannequin …Qty 10 CPR Training mannequin - The mannequin should have simple face shield lung system without need of cleaning, disinfecting or disassembly required - The head of the offered mannequin must be able to tilt/chin lift for opening the airway, tough vinyl skin and a foam-filled torso for resistance to compressions - Must have palpable landmarks to ensure proper hand placement - Must have observable chest rise that verifies ventilation - Must have rate light that comes on when the correct rate is achieved - Must have a second light that comes on to show correct depth with proper release Possibility for assessment of quality of CPR when both lights are on - Must be delivered with three airways and instructions - The depth and rate of the offered Manikin must be set according to internationally accepted standards for BLS -The visual training assistant box to be provided with light indicators that runs on a 9V DC battery |  |  |  |
| I.3 | Training AEDs Qty 5 Configurable in semi-automatic or fully automatic mode Power supply: 1.5V cells or rechargeable batteries It should have metronome Operation should be identical to the real devices, display with instructions and vocal messages It should have minimum 9 preset scenarios Infrared remote control, enabling the instructor to choose and control a scenario Training electrodes, suitable for any mannequin Reusable cable To be delivered with: - 1 infra-red remote control 4 batteries - 3 pairs of training electrodes - 1 reusable cable - 1 casing for batteries |  |  |  |
| I.4 | ILS training mannequin Qty 4 Immediate Life Support training mannequin Product requested features: - To have realistic anatomy including head tilt, chin lift, compression depth, compression force and chest rise. - To have a sensor that indicates correct hand placement - To have a ventilation system that provides appropriate chest rise with Bag Valve Mask and Mouth to Mouth. - Enhanced measurement and feedback capabilities. It must have mode for AED Training that is compatible with the offered AED trainers - To be able to upgrade with wireless connectivity with mannequin controller or software - Guidelines compliant -T be delivered with carrying case |  |  |  |

|  | LOT 2. EQUIPMENT FOR HOSPITAL PATIENT MANAGEMENT |  |  |  |
| --- | --- | --- | --- | --- |
| II. 1 | PATIENT MONITOR WITH CENTRAL MONITORING SYSTEM Qty. 1 set (total 7 monitors and 1 CS) PATIENT MONITOR WITH CENTRAL MONITORING SYSTEM 6x monitors 15” with ECG, resp, SpO2, NIBP I Temp with all peripheral accessories 1x monitor 15” with ECG, resp, SpO2, NIBP I Temp, IBP I CO, with all peripheral accessories for all modules including complete set for measurement of IBP and CO with peripheral sensors 1x Central station for integration of the offered monitors with included installation and calibration For monitoring of adults, children and neonates Touch screen display with min 15”, resolution display from minimum 1366 x 768 To have minimum 11 waveforms displayed To have option for HL7 output Viewing angle from minimum ± 15 degrees Option for full data disclosure of min 48 hours Built-in batteries for at least 12 hours continuous monitoring Weight of the monitor, with no battery of maximum 6.7 kg Dual temperature IBP measuring range from -50 to 300mmHg Measuring NIBP with included minimum 3 types of cliffs Heart rate measuring range from 15-300 bpm SPO2 with minimum 1 sensor for adults and minimum 1 sensors for pediatric use 3/5 lead ECG, etCO2 measure Dual speaker Central monitoring system CMS The easy-to-use and lets you centrally monitor the vital signs on the patients connected System, PC workstation, Keyboard, Mouse, Display Installation and all needed cables for connection included in delivery |  |  |  |
| II.2 | Electrocardiograph - ECG 12 channel Qty 3 12 channel ECG for recording and printing of all 12 channel LCD display with a minimum of 2х16 characters and a pulse display The ability to enter the name, age, gender, height and weight of the patient Combined alphanumeric and functional keyboard Indication contact for each electrode separately Thermal printer with a minimum paper width of 204mm with the ability to print on plain thermal fax paper and make a millimeter template with automatic interpretation of the record Sensitivity of 5, 10 and 20 mm / mV Print speed of 12.5, 25 and 50 mm / s Adaptable filters from 50 to 60 Hz Filter for muscle tremors from 25 to 35 Hz Filters to stabilize the baseline from 0.1Hz -3dB or lower Able to record long ECG Protection from defibrillation The offered device must have a built-in battery Ability to upgrade with ECG computer tracking software Instructions for handling the medical device in Macedonian language. |  |  |  |
| II.3 | Blood glucose, total cholesterol and triglycerides meters Qty. 2 Diagnostic aid multiparameter with capillary aspiration for the determination of the following paramenters: Glucose, Cholesterol, Triglycerides Possibility to execute several determinations from fingerprint simply by changing strip. To be able to determinate glucose, cholesterol and triglycerides monitoring in whole blood. The offered device should be easy to use with external data-chip insertion, -Wide Display, -Automatic switch on by strip insertion Indication of the average glucose level of time periods -Easy cleaning of optical part by the lower cover opening Data Transfering to PC The device should be delivered with Glucose strips, Cholesterol strips and Triglycerides strips |  |  |  |
| II.4 | Portable Digital Ultrasonic Color Doppler premium class with accompanying trolley and probes Qty. 1 Maximum weight of the device 7.5 kg Should have full mechanical alphanumeric QUERTY keyboard integrated in the control panel The screen should be high resolution minimum 15 "LCD screen The device should support probe frequencies ranging from 1 to 15 MHz or more The device must support the following probe technology: "single crystal" and matrix probes Scanning depth should be minimum 30 cm Dynamic range minimum 170 dB Minimum 500,000 digital channels Minimum 500 GB hard disk drive The device must have one connector for an image probe and one connector for non-imaging probes The device must have a software for use in adult cardiology and examination of the vascular system and lung examination The device must be equipped with a battery that allows a minimum autonomy of the device for at list 40 minutes without connection to the power source, and quick charge of the battery Minimum frame rate of 750 frames/second in 2D mode The device must support the following modes of operation: 2D, M-mod, anatomical M-mod, Color M-mod, Doppler color, high-sensitivity color Doppler mode, directional high-sensitivity color Doppler mode, Tissue Doppler, Color Doppler, PW Tissue Doppler, CW Doppler, PW Doppler; Duplex and Triplex mode The device must support an image processing technique in order to improve contrast resolution, reduce artifacts and improve border definitions without changing image resolution at pixel level in real time. Technique must be available on the offered probes (sector and linear probe) The device must have the ability to operate algorithm that enables multidimensional imaging in real time, "Compound Imaging" -or equivalent with a minimum of 9 acquisition images that give a high resolution image after assembly. Continuous automatic gain (gain) and TGC in real time to achieve optimum brightness of the tissue In PW mode, sample window size must be from 0.8 mm to 24 mm or more In CW mode, the speed range is at least 20 m/s Mandatory Directional Power Doppler mode Minimum 2 USB ports on the control panel Possibility of upgrading DICOM 3.0 (storage, worklist, PPS) software Minimum one 3-channel ECG connection Possibility of automatic Doppler signal analysis The ability to upgrade with the program for performing stress echocardiography (to support at least 8 stages) Possibility of upgrading the device with a program for the advanced quantification of the mitral valves. 3D Valve Anatomy Valuation, which allows multiplane reconstruction (3D slipping) and 2D / 3D measurements such as distance, curvature, volume, angles, etc. Possibility of Excel data export or DICOM export Ability for upgrade with software that allows volume display (3D) and full-volume display (4D) using a transezofagial probe Ability for upgrade with software that enables simultaneous display of the transezofagial probe real-time images with supported Color Doppler Possibility of upgrading with software for automated quantification of 2D strains using the "speckle tracking" method Possibility of upgrading with a program that allows the use of contrast in cardiac examination of the left ventricle (with a clip recording time of min 420 seconds), available on the offered sectoral and linear probes The possibility of upgrade with transezofagial probe (TEE) with minimum frequency range of 2 to 7 MHz, made in "single crystal" and matrix technology with min. 2500 crystal elements for 2D, 3D and volume real-time display (4D), with mechanical scaling of scanning angle on the probe from 0 ° to 180 °, that supports modes 2D, harmonic, M-mod, color M-mod, PW Doppler, CW Doppler, in real-time comparison of color images and 2D images, full-volume display (4D) The device must support following probes: sectoral (TTE and TEE), abdominal, linear, intraoperative (abdominal and linear) as well as probe for Intracardial Echocardiography (ICE) To be delivered with a trolley compatible with the device: - Maximum weight of 48 kg - Possibility to adjust trolley in height from 90cm to 108cm - minimum 3 active connectors for imaging ultrasound probes Sector probe: - Broadband sector probe; supports "single crystal" technology, a minimum frequency range of 1 to 5 MHz or more; with minimum 80 crystal elements; Scanning widths must be max. 21 mm. Supports 2D, harmonics, PW Doppler, CW Doppler, Color Doppler modes. Linear probe: - Broadband linear probe, minimum frequency range from 3 to 12MHz (+/- 0.5MHz), with min. 160 crystal elements, width min. 38mm. Supports 2D, harmonic, HPRF, PW Doppler. The probe must support needle guide |  |  |  |